I'll help you create a comprehensive testing guide for NTP functionality in the prplOS framework. Let me search for the relevant information from the prpl foundation and their GitLab repositories.Based on my research of the prpl Foundation, prplOS framework, and related documentation, I'll provide you with a comprehensive professional testing guide for NTP feature functionality on prplOS.

# NTP Feature Functionality Testing Guide for prplOS Framework

## Executive Overview

prplOS is an open-source operating system designed for embedded networking devices, particularly broadband routers and gateways, built on OpenWRT that leverages modern microservices and containerized applications. It implements the TR-181 Data Model from the Broadband Forum, enabling cross-platform portability of applications and services.

## 1. Architecture Overview

### 1.1 Core Components

Time Management Architecture:

- TR-181 Device.Time Object: Standardized data model for time configuration

- chronyd Process: Modern NTP client/server implementation (replaces legacy ntpd)

- UCI Configuration System: OpenWRT's Unified Configuration Interface

- System Configuration Files:

  - /etc/config/system - System-level timezone and NTP settings

  - /etc/config/chrony - chronyd-specific NTP configuration

  - /etc/chrony.conf - Runtime configuration for chronyd

### 1.2 TR-181 Device.Time Data Model

The Device.Time object contains these key parameters:

| Parameter | Type | Access | Description |
| --- | --- | --- | --- |
| Enable | boolean | R/W | Enables/disables NTP client functionality |
| Status | string | R | Current status: "Disabled", "Unsynchronized", "Synchronized", "Error" |
| NTPServer1-5 | string | R/W | NTP server addresses (hostname or IP) |
| LocalTimeZone | string | R/W | POSIX timezone format (e.g., "EST5EDT,M3.2.0,M11.1.0") |
| CurrentLocalTime | dateTime | R | Current local time with timezone applied |

## 2. Test Environment Setup

### 2.1 Prerequisites

Hardware Requirements:

- prplOS-compatible device (Freedom board, MediaTek reference platform, or compatible gateway)

- Network connectivity with internet access

- Serial console access for debugging

Software Requirements:

- prplOS v3.1.0 or later

- SSH client for remote access

- NTP testing tools: ntpdate, chronyc, timedatectl

Network Requirements:

- Access to public NTP servers (pool.ntp.org) or internal NTP infrastructure

- UDP port 123 open for NTP traffic

- DNS resolution working properly

### 2.2 Access Methods

# SSH Access

ssh root@<device-ip>

# Serial Console

screen /dev/ttyUSB0 115200

# Web UI (if enabled)

https://<device-ip>

## 3. TEST CASE 1: NTP Server Default Settings

### 3.1 Objective

Verify default NTP server configuration after fresh installation and validate proper time synchronization.

### 3.2 Test Procedure

#### Step 1: Check Default System Configuration

# Check UCI system configuration

uci show system.ntp

# Expected output should show:

# system.ntp=timeserver

# system.ntp.enabled='1'

# system.ntp.enable_server='0'

# system.ntp.server='0.openwrt.pool.ntp.org'

# system.ntp.server='1.openwrt.pool.ntp.org'

# system.ntp.server='2.openwrt.pool.ntp.org'

# system.ntp.server='3.openwrt.pool.ntp.org'

#### Step 2: Verify chronyd Configuration

# Check chronyd UCI configuration

uci show chrony

# View actual chronyd configuration

cat /etc/chrony.conf

# Check if chronyd is running

ps | grep chronyd

# Verify chronyd service status

/etc/init.d/chronyd status

#### Step 3: Verify NTP Server List

# Using chronyc to check configured sources

chronyc sources

# Expected output format:

# MS Name/IP address         Stratum Poll Reach LastRx Last sample

# ===============================================================================

# ^* 0.openwrt.pool.ntp.org      2   6   377    34   +123us[ +456us] +/-   15ms

# ^+ 1.openwrt.pool.ntp.org      1   6   377    35   -234us[ -567us] +/-   22ms

#### Step 4: Check TR-181 Parameters via CWMP

If TR-069/USP management is enabled:

# Query TR-181 parameters

dmcli eRT getv Device.Time.

# Verify:

# Device.Time.Enable = true

# Device.Time.Status = "Synchronized"

# Device.Time.NTPServer1 = "0.openwrt.pool.ntp.org"

# Device.Time.NTPServer2 = "1.openwrt.pool.ntp.org"

#### Step 5: Verify Time Synchronization

# Check chronyd tracking information

chronyc tracking

# Expected information:

# - Reference ID and name

# - System time offset

# - Last offset and RMS offset

# - Frequency of system clock

# - Stratum level

# Check system time

date

# Verify hardware clock

hwclock -r

### 3.3 Expected Results

✅ Pass Criteria:

- Default NTP servers are pool.ntp.org variants (0-3.openwrt.pool.ntp.org)

- NTP client is enabled by default

- chronyd process is running

- Device.Time.Enable = true

- Device.Time.Status = "Synchronized" (after initial sync)

- System time is within 1 second of actual UTC time

- At least one NTP source shows "^*" (selected) or "^+" (candidate) in chronyc sources

❌ Fail Criteria:

- No NTP servers configured

- chronyd not running

- Device.Time.Status shows "Error" or "Unsynchronized" for >5 minutes

- Time offset > 1 second after synchronization

### 3.4 Troubleshooting

# If synchronization fails:

# 1. Check network connectivity

ping -c 3 0.openwrt.pool.ntp.org

# 2. Verify DNS resolution

nslookup 0.openwrt.pool.ntp.org

# 3. Check firewall rules

iptables -L -n | grep 123

# 4. Review chronyd logs

logread | grep chronyd

# 5. Force immediate synchronization

chronyc makestep

# 6. Restart chronyd service

/etc/init.d/chronyd restart

## 4. TEST CASE 2: NTP Timezone Default Settings

### 4.1 Objective

Verify default timezone configuration and proper local time calculation.

### 4.2 Test Procedure

#### Step 1: Check Default Timezone Configuration

# View system timezone settings

uci show system.@system[0] | grep zone

# Expected output:

# system.@system[0].timezone='UTC'

# system.@system[0].zonename='UTC'

# Alternative method

cat /etc/TZ

# Check symbolic link to zoneinfo

ls -l /etc/localtime

#### Step 2: Verify Current System Time

# Check date with timezone

date

# Expected format: Wed Oct  8 14:30:45 UTC 2025

# Using timedatectl (if available)

timedatectl status

# Expected output includes:

# Local time: Wed 2025-10-08 14:30:45 UTC

# Universal time: Wed 2025-10-08 14:30:45 UTC

# Time zone: UTC (UTC, +0000)

#### Step 3: Verify TR-181 LocalTimeZone Parameter

# Query via CWMP interface

dmcli eRT getv Device.Time.LocalTimeZone

# Default should be:

# Device.Time.LocalTimeZone = "UTC" or "UTC0"

# Query CurrentLocalTime

dmcli eRT getv Device.Time.CurrentLocalTime

# Should show current time in configured timezone

#### Step 4: Test Timezone Change

# Change timezone to test configuration

uci set system.@system[0].timezone='EST5EDT,M3.2.0,M11.1.0'

uci set system.@system[0].zonename='America/New_York'

uci commit system

# Restart system time service

/etc/init.d/system restart

# Verify timezone applied

date

# Should now show EDT/EST timezone

# Verify TZ environment variable

echo $TZ

#### Step 5: Verify Timezone Persistence

# Reboot device

reboot

# After reboot, verify timezone maintained

date

uci show system.@system[0] | grep zone

### 4.3 Expected Results

✅ Pass Criteria:

- Default timezone is UTC

- System time correctly displays in UTC

- Device.Time.LocalTimeZone correctly reports configured timezone

- Device.Time.CurrentLocalTime shows accurate local time

- Timezone changes take effect after system restart

- Timezone configuration persists after reboot

❌ Fail Criteria:

- Incorrect default timezone

- Local time calculation errors

- Timezone changes don't apply

- Configuration lost after reboot

### 4.4 Common Timezone Formats (POSIX)

# UTC

timezone='UTC'

# US Eastern Time (with DST)

timezone='EST5EDT,M3.2.0,M11.1.0'

# US Pacific Time (with DST)

timezone='PST8PDT,M3.2.0,M11.1.0'

# Central European Time (with DST)

timezone='CET-1CEST,M3.5.0,M10.5.0/3'

# India Standard Time (no DST)

timezone='IST-5:30'

# Japan Standard Time (no DST)

timezone='JST-9'

## 5. TEST CASE 3: Daylight Saving Time (DST) Feature Testing

### 5.1 Objective

Verify automatic DST transitions and proper time adjustment during DST changes.

### 5.2 Understanding POSIX DST Format

The POSIX timezone format for DST is: STD offset DST [offset],start[/time],end[/time]

Example: EST5EDT,M3.2.0,M11.1.0

- EST5: Standard time (EST = UTC-5)

- EDT: Daylight time name

- M3.2.0: Start DST on 2nd Sunday (0) of March (3) at 2:00 AM

- M11.1.0: End DST on 1st Sunday (0) of November (11) at 2:00 AM

### 5.3 Test Procedure

#### Step 1: Configure Timezone with DST

# Set US Eastern timezone with DST

uci set system.@system[0].timezone='EST5EDT,M3.2.0/2,M11.1.0/2'

uci set system.@system[0].zonename='America/New_York'

uci commit system

/etc/init.d/system restart

# Verify configuration

date

cat /etc/TZ

#### Step 2: Check Current DST Status

# Using timedatectl (if available)

timedatectl status

# Look for:

# DST active: yes/no

# Last DST change: <date and time>

# Next DST change: <date and time>

# Alternative: check date output

date '+%Z %z'

# During DST: EDT -0400

# Outside DST: EST -0500

#### Step 3: Simulate DST Transition

Method 1: Manual Time Change (for testing)

# Save current time

CURRENT_TIME=$(date +%s)

# Set time to 1 hour before DST spring forward (2nd Sunday March 1:59 AM)

# Example: March 9, 2025 01:59:00 EST

date -s "2025-03-09 01:59:00"

# Wait or manually advance time

sleep 120

# Check if time jumped to 3:01 AM EDT

date

# Expected: Sun Mar  9 03:01:00 EDT 2025

# Restore original time

date -s @$CURRENT_TIME

Method 2: Test with chrony's makestep

# Set system time near DST boundary

date -s "2025-03-09 01:55:00"

# Let chronyd synchronize (will include DST logic)

chronyc makestep

# Monitor time transition

watch -n 1 date

#### Step 4: Verify DST Transition Logging

# Check system logs for DST events

logread | grep -E "time|DST|timezone"

# Check chronyd logs

logread | grep chronyd

# Expected entries:

# - Time zone change notifications

# - Clock step events during transitions

#### Step 5: Test Fall Back Transition

# Set time to 1 hour before DST fall back (1st Sunday November 1:59 AM)

# Example: November 2, 2025 01:59:00 EDT

date -s "2025-11-02 01:59:00"

# Wait for transition

sleep 120

# Check if time fell back to 1:01 AM EST

date

# Expected: Sun Nov  2 01:01:00 EST 2025 (time repeated)

#### Step 6: Verify TR-181 Parameters During DST

# Check if CurrentLocalTime reflects DST

dmcli eRT getv Device.Time.CurrentLocalTime

dmcli eRT getv Device.Time.LocalTimeZone

# Compare with system date

date -Iseconds

# Times should match including timezone offset

#### Step 7: Test DST with NTP Synchronization

# Ensure chronyd is synchronizing

chronyc sources -v

# Set time near DST boundary

date -s "2025-03-09 01:58:00"

# Force NTP sync

chronyc makestep

# Monitor chronyd behavior

chronyc tracking

# Verify smooth transition without time jumps

### 5.4 DST Test Scenarios

#### Scenario A: Spring Forward (DST Begins)

# Test Case: 2 AM → 3 AM transition

# Timeline:

# 01:59:59 EST → 03:00:00 EDT (skip 02:00-02:59)

# Verification Points:

# 1. No times between 2:00-2:59 AM exist

# 2. Clock advances 1 hour instantly

# 3. Timezone abbreviation changes from EST to EDT

# 4. UTC offset changes from -0500 to -0400

# 5. Services continue operating normally

# 6. chronyd adjusts without errors

#### Scenario B: Fall Back (DST Ends)

# Test Case: 2 AM → 1 AM transition

# Timeline:

# 01:59:59 EDT → 01:00:00 EST (repeat 01:00-01:59)

# Verification Points:

# 1. Hour from 1:00-1:59 AM occurs twice

# 2. Timezone abbreviation changes from EDT to EST

# 3. UTC offset changes from -0400 to -0500

# 4. System handles ambiguous timestamps correctly

# 5. Logs show proper transition

# 6. No service disruption

### 5.5 Expected Results

✅ Pass Criteria:

- Timezone correctly configured with DST rules

- Spring forward: time jumps from 1:59:59 to 3:00:00

- Fall back: time repeats 1:00:00-1:59:59

- Timezone abbreviation changes (EST ↔ EDT)

- UTC offset changes appropriately (-0500 ↔ -0400)

- Device.Time.CurrentLocalTime reflects DST-adjusted time

- chronyd continues synchronization through transitions

- No service crashes or time-related errors

- System logs show DST transition events

❌ Fail Criteria:

- DST transition doesn't occur

- Incorrect time after transition

- Services crash during transition

- Time jumps in wrong direction

- Timezone offset incorrect

- chronyd loses synchronization

### 5.6 Advanced DST Testing

#### Test Different DST Rules

# European DST (Last Sunday March/October)

uci set system.@system[0].timezone='CET-1CEST,M3.5.0,M10.5.0/3'

# Australian DST (First Sunday October, First Sunday April)

uci set system.@system[0].timezone='AEST-10AEDT,M10.1.0,M4.1.0/3'

# No DST regions

uci set system.@system[0].timezone='IST-5:30'  # India

uci set system.@system[0].timezone='JST-9'     # Japan

#### Stress Test: Rapid DST Changes

#!/bin/sh

# Script to test multiple DST transitions

for i in $(seq 1 10); do

echo "Test iteration $i"

# Set to before DST spring forward

date -s "2025-03-09 01:59:00"

sleep 2

# Verify transition

AFTER_DST=$(date '+%Z')

echo "After spring: $AFTER_DST"

# Set to before DST fall back

date -s "2025-11-02 01:59:00"

sleep 2

# Verify transition

AFTER_FALL=$(date '+%Z')

echo "After fall: $AFTER_FALL"

# Restore NTP sync

chronyc makestep

sleep 5

done

### 5.7 Troubleshooting DST Issues

# If DST not working:

# 1. Verify POSIX timezone format is correct

cat /etc/TZ

# Check syntax: STDoffsetDST,start,end

# 2. Check zoneinfo files (if using zonename)

ls -l /usr/share/zoneinfo/

# 3. Verify system date utility supports DST

date --help | grep -i dst

# 4. Test timezone manually

TZ='EST5EDT,M3.2.0,M11.1.0' date

# 5. Check for conflicting time settings

uci show system | grep time

uci show chrony

# 6. Restart all time-related services

/etc/init.d/system restart

/etc/init.d/chronyd restart

## 6. Comprehensive Integration Testing

### 6.1 End-to-End Test Scenario

#!/bin/sh

# Complete NTP functionality test script

echo "=== prplOS NTP Feature Test Suite ==="

# Test 1: Verify default NTP configuration

echo "\n[TEST 1] NTP Server Default Settings"

uci show system.ntp

chronyc sources

echo "RESULT: $(chronyst sources | grep -c '^[\^]' > 0 && echo PASS || echo FAIL)"

# Test 2: Verify timezone configuration

echo "\n[TEST 2] Timezone Default Settings"

uci show system.@system[0] | grep zone

date

echo "RESULT: $(date '+%Z' | grep -q 'UTC\|EST\|PST' && echo PASS || echo FAIL)"

# Test 3: Test DST functionality

echo "\n[TEST 3] DST Feature Test"

CURRENT_TZ=$(date '+%Z')

echo "Current timezone: $CURRENT_TZ"

# Configure DST-enabled timezone

uci set system.@system[0].timezone='EST5EDT,M3.2.0,M11.1.0'

uci commit system

/etc/init.d/system restart

sleep 3

NEW_TZ=$(date '+%Z')

echo "New timezone: $NEW_TZ"

echo "RESULT: $([ -n "$NEW_TZ" ] && echo PASS || echo FAIL)"

# Test 4: NTP synchronization

echo "\n[TEST 4] NTP Synchronization"

chronyc tracking

SYNC_STATUS=$(chronyc tracking | grep "Leap status" | awk '{print $4}')

echo "Sync status: $SYNC_STATUS"

echo "RESULT: $([ "$SYNC_STATUS" = "Normal" ] && echo PASS || echo FAIL)"

echo "\n=== Test Suite Complete ==="

### 6.2 Performance Metrics

Monitor these metrics during testing:

# Time to first sync

chronyc tracking | grep "Last offset"

# Sync accuracy

chronyc tracking | grep "System time"

# Stratum level

chronyc tracking | grep "Stratum"

# Network latency to NTP servers

chronyc sources -v | grep "Offset"

# CPU usage

top -bn1 | grep chronyd

## 7. Configuration Management

### 7.1 Backup Configuration

# Backup all time-related configs

tar -czf ntp-config-backup.tar.gz \

/etc/config/system \

/etc/config/chrony \

/etc/chrony.conf \

/etc/TZ

# Restore configuration

tar -xzf ntp-config-backup.tar.gz -C /

uci commit

/etc/init.d/system restart

/etc/init.d/chronyd restart

### 7.2 Factory Reset Testing

# Perform factory reset

firstboot

reboot

# After reboot, verify defaults restored:

# - NTP servers back to pool.ntp.org

# - Timezone back to UTC

# - chronyd running with default config

## 8. Compliance and Validation

### 8.1 TR-181 Compliance Checklist

- [ ] Device.Time.Enable parameter functional

- [ ] Device.Time.Status accurately reflects sync state

- [ ] Device.Time.NTPServer1-5 configurable

- [ ] Device.Time.LocalTimeZone supports POSIX format

- [ ] Device.Time.CurrentLocalTime returns correct local time

- [ ] All parameters persist across reboots

- [ ] CWMP/USP operations supported

### 8.2 Broadband Forum Requirements

Based on BBF TP-181 specifications, NTP testing should verify that the ACS can set NTPServer parameters, the CPE communicates with configured servers, and time synchronization occurs after reboot.

## 9. Common Issues and Solutions

| Issue | Symptom | Solution |
| --- | --- | --- |
| No NTP sync | Status stays "Unsynchronized" | Check network, DNS, firewall; verify server reachability |
| Wrong timezone | Time offset incorrect | Verify POSIX timezone format; check /etc/TZ file |
| DST not working | Time doesn't change at DST boundary | Verify DST rules in timezone string; check system date support |
| chronyd crash | Process not running | Check logs; verify config syntax; restart service |
| Time drift | Clock loses accuracy | Check RTC battery; verify stratum of NTP sources |

## 10. Documentation References

- prpl Foundation: https://prplfoundation.org/

- prplOS GitLab: https://gitlab.com/prpl-foundation/prplos/prplos

- Broadband Forum TR-181: https://www.broadband-forum.org/technical/download/TR-181.pdf

- OpenWRT Time Sync Docs: https://openwrt.org/docs/guide-user/services/ntp/client-server

- chrony Documentation: https://chrony.tuxfamily.org/documentation.html

## Conclusion

This comprehensive testing guide covers all aspects of NTP functionality testing in the prplOS framework, including server configuration, timezone management, and DST handling. Follow these procedures systematically to ensure robust time synchronization in your prplOS deployment.

For additional support, refer to the prpl Foundation community forums or submit issues to the prplOS GitLab repository.